# Cruise Report

R/V Endeavor, Cruise EN651

27 February 2020 – 17 March 2020

Bridgetown, Barbados to Praia, Cape Verde

Chief Scientist

Rainer Lohmann

Principle Investigators, NSF Grant OCE-1433150

Rainer Lohmann, Robert A. Pockalny

Shipboard Science Party

Benjamin Geyman (Harvard), Samuel Katz (GSO/URI), Rainer Lohmann (GSO/URI), Gabe Matthias (GSO/URI Marine Technician), Nuria Penalva Arias (U.A. Barcelona), Robert Pockalny (GSO/URI), Antoni Rosell-Mele (U.A. Barcelona), Lydia Sgouros (GSO/URI Marine Technician, intern).

Images of CTD rosette casts (left) and multi-corer operations (center) and hi-vol samplers (right) aboard the R/V Endeavor. Photos by Sam Katz, GSO/URI graduate student.

Ship’s track of the R/V Endeavor with coring and sampling stations occupied during EN651.

Table of Contents

Executive Summary……………………………………………..............................	3

I.  Introduction………………………………………………………………….…	4

II.  Operations……………………………………………………………...………	7

Overview……………..…………………………………...……………..……	7

Station Selection……………..…………………………………...………	9

CTD and Niskin Water Sample Casts……………………………………...…...…	9

Surface Water Samples………………………………………………...…...…

Sediment Samples……………………………………………………………...…...…

Air Samples……………………………………………………………………

References………………………………………………………………….………

Appendices (each in a separate file)

A.	Scientist Affiliations

B.	Site Summaries

C.	Core Photos

D.	CTD / Rosette Operations

E.	Autonomous University of Barcelona Participant Operations

F.	Marine Technician Operations

Executive Summary

During Cruise EN651 we occupied 15 different stations along a west-to-east transect of the equatorial Atlantic centered on about 5˚N and successfully collected sediment samples from 12 multicore deployments, water samples/water column measurements from 16 CTD rosette casts, and near-continuous air and surface water sampling.  Our sites included a variety of sediment deposition environments including the Amazon Submarine Fan, a series of sites across the Mid-Atlantic Ridge, and sites on the Sierra Leone Rise and adjacent Sierra Leone Basin.

We successfully achieved our primary and most important goals of collecting sediment, water samples, and air samples along a west-to-east transect in the tropical Atlantic centered on about 5˚N.  In addition, we also were able to collect similar samples along a roughly south-to-north transect along the eastern edge of the Atlantic along the 20˚ W meridian within about 500 km from the African coast.  Planned coring locations were spaced equidistant across the west-to-east transect; however, actual stations were identified with brief sub-bottom surveys to ensure sediment sampled was characteristic of the region and/or primarily pelagic in origin (e.g., on flat-lying sediments away from potential mass wasting vents or turbidite flow).  CTD and water-sampling casts were conducted at all but one station (e.g., 3b) and varied between casts to depths of 1000 m and full-ocean depth (e.g., ~ 4000 m).  Air and surface water sampling for particulate matter were collected only while the ship was underway and when the apparent wind was within 45˚ of the ship’s bow.

I.  Introduction

This research cruise was the result of a successful proposal to the US National Science Foundation, Division of Ocean Sciences (Proposal Number OCE-1924191). The lead Principle Investigators (Lohmann and Pockalny) requested support to collect sediments, water particles, and air particles to characterize the concentrations and sources of black carbon across tropical Atlantic air and sediment.

These samples will be used to test a primary hypothesis that sedimentary black carbon across the tropical Atlantic originates from biomass burning and is mainly delivered via atmospheric deposition.  Anticipated observations and predictive consequences in support of this hypothesis include

1) sedimentary black carbon in the tropical Atlantic is 14C-modern

2) atmospheric black carbon particles in the marine boundary layer of the tropical Atlantic are 14C -enriched and originate from biomass burning

3) overall black carbon concentrations will decrease with increasing distance from Africa and contain relatively more soot black carbon.

The cruise achieved the primary goal of collecting sediment cores, water samples, and air samples along a west-to-east transect in the tropical Atlantic centered on about 5˚N (Fig. 1, Table I).  In addition, we also were able to collect similar samples along a roughly south-to-north transect along the eastern edge of the Atlantic along the 20˚ W meridian within about 500 km from the African coast.

We collected multi-cores from 12 stations with recovery of at the sediment-water interface extending to depths of 30 to 80 cm.  A total of 16 CTD rosette casts at all but one coring station (e.g., station 3b) were deployed to depths of 1000 m and/or full-ocean depth (e.g., ~4000 m).  A total of 12 water samples were taken at each CTD cast at varying depths depending on water column properties.  Near-surface water samples were taken and filtered while the ship was underway.  Air samples with high volume air samplers were obtained while the ship was underway and apparent wind direction conditions were appropriate (e.g., within 45˚ of the bow).

Figure 1. Location map of coring stations overlain on sediment type (Dutkiewicz et al., 2015). White dots indicate successful sediment coring sites and black dots indicate unsuccessful coring sites. Vertical black line indicates sites of deep CTD. Ship’s track is shown as thick dotted line, EEZs are shown as thin dotted lines, and 1000-m bathymetry contours are shown as white lines.

Table I. General information for each station occupied by EN651. Events in gray indicate no sediments recovered in the core. Events with a * indicate sediment samples obtained from scrapings on corer. Events with a ^ indicate aborted deployments.

Station	Longitude	Latitude	Depth	Start Station (UTC)	End Station (UTC)	|	Events

(+E/-W)	(+N/-S)	  (m)	(mm/dd/yy hh:mm)	(mm/dd/yy hh:mm)	|

EN651-01	-48˚ 08.7'	07˚ 25.0'	4250		03/01/20 13:19		03/02/20 00:15	|	CTD01	MC01	MC02

EN651-02	-44˚ 27.0'	05˚ 29.0'	3310		03/03/20 02:29		03/03/20 15:28	|	CTD01	MC01*	MC02	BC01

EN651-03	-40˚ 21.8'	05˚ 02.4'	4645		03/04/20 17:48		03/05/20 04:52	|	CTD01	MC01	MC02

EN651-03b	-38˚ 58.8'	04˚ 59.8'	4660		03/05/20 13:07		03/05/20 17:38	|	MC01

EN651-04	-35˚ 47.2'	05˚ 59.9'	3990		03/06/20 13:27		03/06/20 20:28	|	CTD01(deep)	MC01

EN651-05	-33˚ 36.6'	05˚ 00.2'	3730		03/07/20 11:35		03/07/20 21:29	|	CTD01(deep)	MC01^	MC02^	MC03

EN651-06	-30˚ 03.4'	04˚ 59.8'	3605		03/08/20 19:46		03/09/20 02:01	|	CTD01(deep)	MC01

EN651-07	-27˚ 00.0'	05˚ 09.9'	4275		03/09/20 19:46		03/10/20 01:09	|	CTD01	MC01

EN651-08	-23˚ 26.5'	05˚ 00.0'	4320		03/10/20 22:24		03/11/20 04:16	|	CTD01	MC01

EN651-09	-20˚ 00.0'	05˚ 30.1'	2840		03/11/20 22:24		03/12/20 04:24	|	CTD01	MC01

EN651-10	-17˚ 30.0'	05˚ 30.0'	4965		03/12/20 18:17		03/13/20 00:17	|	CTD01	MC01

EN651-11	-17˚ 30.0'	07˚ 30.0'	4820		03/13/20 11:51		03/13/20 17:50	|	CTD01	MC01

EN651-12	-20˚ 10.8'	07˚ 59.2'	4180		03/14/20 10:30		03/14/20 19:27	|	CTD01(deep)	MC01	CTD02

EN651-13	-21˚ 00.0'	10˚ 22.5'	5130		03/15/20 09:20		03/15/20 17:50	|	CTD01	MC01	CTD02

EN651-14	-21˚ 00.0'	12˚ 00.0'	4890		03/16/20 04:00		03/16/20 21:58	|	MC01	CTD01

CTD = hydrocast with CTD and 12-bottle Niskin bottle rosette

MC = multi-corer deployments with Ocean Instruments MC-800

BC = box corer

Background

Black carbon (BC) is a highly graphitized incomplete combustion byproduct that could be a sink for fixed carbon, especially when deposited to pelagic sediments (Kuhlbusch 1998). In the atmosphere, BC is a key driver of global climate change; it is second only to CO2 in its contribution to global warming (Bond et al. 2013; Bond and Sun 2005; Gustafsson and Ramanathan 2016; Ramanathan and Carmichael 2008). There is a general assumption that rivers deliver most or all BC to the ocean (Coppola et al. 2018; Elmquist et al. 2008; Kuhlbusch 1998; Masiello and Druffel 2001; Mitra et al. 2014, 2002; Suman et al. 1997). Yet the effects and fluxes of BC are not well constrained in general circulation models. For example, few BC flux measurements are available in remote ocean sediments due to the expense and difficulty of obtaining samples. The proposed work is thus relevant and timely for our understanding of the carbon cycle, a key component of our ability to forecast climate and its change. Our proposed work is important because the formation of BC during incomplete combustion results in a fraction of carbon not being available for the biological pump and deep ocean respiration. Once deposited to the ocean, BC is buried in sediments, and can account for significant fractions of the organic matter preserved in sediments. Hence, our results would contribute towards accounting some of the ‘missing’ terrestrial OC in sediments, as all BC is landmass derived (Hedges et al. 1997).

Education and Outreach

Cruise EN 651 provided an educational component to three graduate students (Sam Katz, URI; Ben Geyman, Harvard University; and Nuria Penalva, Autonomous University of Barcelona, Catalonia-Spain) one MATE-at-Sea intern (Lydia Sgouros, Case Western, OH). A fourth (Spanish) graduate student was supposed to be trained on the transit back from Cape Verde, but due to concerns about CoVID-19, she was not allowed to board the ship prior to our departure back to Narragansett (RI). The cruise provided an opportunity for all members of the science team to be trained in cross-disciplinary oceanography through the collaboration with complementary science activities and foci.

During the return transit, Lohmann was interviewed by a journalist working for AGU’s EOS magazine about the impact of COVID-19 on seagoing research.

Societal Relevance

Results from the research above are highly relevant to our understanding of the carbon cycle. If soot BC is indeed an important fraction of deep sea sediment, even less marine-produced organic carbon is sequestered in sediments. Currently, the prevailing view is that black carbon is mostly delivered via fluvial discharges to the ocean. Yet our previous results from tropical Atlantic Ocean sediment show very young BC in the sediment, possibly from aeolian deposition, challenging the view of current BC sources to and cycling in the oceans. We will test the extent to which BC is 14C modern across the Atlantic Ocean by concurrently sampling BC in atmosphere, water column and sediment. Our previous results also imply that C4 biomass burning is an important BC source in tropical Atlantic Ocean sediment. The origin of BC in atmosphere, water column and sediment will be assessed by 13C and 14C isotope ratio analysis, and the sources corroborated with organic tracers. To address concerns that a given BC method is not adequate, we will be relying on two complementary BC methods, and thereby strengthen confidence in BC concentrations and fluxes. The proposed research will help close the gap between known BC emissions and sinks.

Data Management

The underway data, CTD casts, and cores from this cruise will be delivered to national data archives and core repositories shortly after the completion of the cruise.  Access to these data will be made available immediately to other researchers.  Derived onboard measurements from sediment, water, and air samples will remain the property of the respective research groups until publication.  Individual research groups will be responsible for uploading the published or unpublished data to the appropriate repository (e.g., SedDB, BCO-DMO, NGDC).

All relevant raw underway geophysical data (e.g., navigation, bathymetry, seismics, and ADCP) data will be immediately uploaded and openly accessible through the R2R program.  No moratorium is anticipated for these raw data.

All meteorological surface measurements (e.g., wind, pressure, temperature, salinity and fluorometry) and water column measurements (e.g., CTD salinity, temperature, fluorometry) will be provided to NOAA or other appropriate repositories for immediate access and use through the NSF-supported R2R program (www.rvdata.us).

Sediment cores will be stored at the NSF-funded Rock and Core Repository at GSO-URI and access to samples will follow established protocols.  Solid material, in the form of cores will be curated and retained after the expedition and made available to other investigators that wish to use them for other means.

Cruise related materials including the cruise report, figure archives, and relevant data will be temporarily hosted on a password-protected server at GSO-URI for access by the cruise participants and the proposal’s PIs.

Shipboard Scientific Party

The shipboard scientific party (names and affiliations in Appendix A) consisted of several groups of individuals grouped according to their expertise and research interest.

Chief Scientist:			Lohmann

Site Selection:			Pockalny

Sediment Samples:		Lohmann, Katz, Geyman, Rosell-Mele, Penalva Arias

CTD Casts:			Geyman, Rosell-Mele, Penalva Arias

Water Column Samples:	Geyman, Rosell-Mele, Penalva Arias

Surface Water Samples: 	Katz, Lohmann

Air Samples:			Lohmann, Katz, Rosell-Mele, Penalva Arias

II.  Operations

### Overview

Our proposed cruise plans requested 20 days at sea and assumed start and end ports of Bridgetown, Barbados and Dakar, Senegal, respectively.  These plans included 5.5 days of transit to/from our operational area, 8.5 days of transit between coring sites, and 6 days on station at 10 coring sites along a west-to-east transect centered on 5˚N.  Our planned on-station order of events included a 3-hour site survey, a CTD rosette cast to 1000 m, and two multi-corer deployments for an average on-station time of 0.6 days.  We assumed 30 m/min wireline speeds for both the CTD casts and multi-corer deployments.

Our actual cruise aboard the R/V Endeavor began in Bridgetown, Barbados on February 27th, 2020, but the end port was changed to Praia, Cape Verde for security purposes and the cruise ended on March 17th, 2020.  We departed Barbados 3 days later than anticipated due to issues with a ship’s generator; however, our cruise was extended to provide us with the requested 20 days at sea.

Our actual order of events generally followed our proposed plan, but the actual time allocations differed (Fig. 2).  Our site survey time and CTD casts required less time than planned, even though 4 casts were extended to ~4000 m depth and there were multiple casts at 2 sites.

Our coring time was significantly reduced after Station 3b once the multi-corer was fine-tuned and only one deployment was required to obtain the required sediment samples.

The much-improved efficiency of on-station operations allowed us to occupy 4 additional sites and begin a roughly south-to-north transect located about 500 km offshore of Africa between Sierra Leone Rise and Cape Verde Plateau.  A detailed timeline of cruise operations is provided in Figure 3.

We should also note that the cruise occurred during the COVID-19 pandemic in early 2020 and required rerouting of the R/V Endeavor back to its home port in Narragansett, RI after the cruise.  We had originally planned to continue air and water sampling as the ship transited to Port Everglades FL for its next cruise, but those plans have been modified.  Most of the science participants were allowed to remain on the vessel for the transit back due to cancelled flights; however, our two colleagues from U.A. Barcelona disembarked in Cape Verde and transited back to Barcelona on a later flight.

Figure 2. Pie charts comparing planned (left) and actual (right) time allocation during the cruise.

Figure 3. Daily timeline of operations during cruise EN651.  Color code is the same as the pie charts in Fig. 2.

### Coring Site Selection (Pockalny)

Objectives.  Our principal geophysical objectives were to locate coring sites and to characterize the tectonic/depositional setting.

Protocol.  These objectives were achieved primarily with Knudsen 320B and 3260 sub-bottom echosounders; however, previously collected multibeam bathymetry and backscatter data also were used to target coring sites.

The sub-bottom data were collected at ship speeds of 7 to 10 knots for about 30 minutes prior to arriving at a target site and then another 30 minutes after leaving the station. The data were recorded in .kea, .keb, and SEGY formats and transferred to a Dell PC to view the data.  The .keb files were converted to compressed .keb formats with Knudsen’s Conversion Utility application and then viewed with the SounderSuite PostSurvey v.4.07 application. Contrast values were adjusted to optimize seismic section images and then saved as bitmap formats.

Previously collected multibeam data were obtained from the NOAA multibeam archive (https://www.ngdc.noaa.gov/mgg/bathymetry/multibeam.html) and then processed with shell scripts using MB-Systems and Generic Mapping Tools.  In regions where multibeam data were not available, we used the GMRT mapping tool (https://www.gmrt.org/GMRTMapTool/) to extract gridded bathymetry data.  The trackline locations of the seismic sections were obtained from the shipboard SCS data files in the 60sec format.

Preliminary Results.  Sub-bottom seismic data and previous multibeam data and satellite-derived topography were combined to generate regional views of the sediment profiles and coring areas (see Appendix B, Site Summaries).

Post-cruise Activities.  Post-cruise activities will include generating site maps of geophysical and environmental data as needed in support of related project activities.

CTD and Niskin Rosette Casts (Benjamin Geyman, Harvard U., Paulson School of Engineering and Applied Sciences)

Objectives.  Harvard’s participation in the EN651 cruise provided an opportunity for graduate training in oceanographic measurement of trace metals and associated clean sampling and handling procedures. Research goals included characterizing: (i) the vertical distribution of mercury and methylmercury species in the epi- and mesopelagic water column, and (ii) examining accumulation of mercury in deep-sea sediments. Seawater mercury distributions have not been extensively studied in the Equatorial Atlantic Ocean, and data collected during EN651 will be used to support evaluation of global ocean mercury transport models.

Protocol.

CTD Configuration

Config file: EN651_0444a.xmlcon

SBE 911plus/917plus CTD

WET Labs ECO-AFL/FL – fluorometer

WET Labs C-Star – transmissometer

SBE 43 – O2

Biospherical/Licor - PAR/Irradiance

SBE Carousel

Ocean Test Equipment – Niskin bottles (10-L)

Seawater was collected from stations 1 – 10 at between the surface and 4,000 m depth using Niskin bottles deployed on a CTD rosette. Seawater collected for Hg analysis was transferred within 1 hour of recovery into 1 L, acid-cleaned glass bottles following “clean hands-dirty hands” techniques outlined in EPA Method 1669 (1996). An additional 1 L sample was collected into plastic bottles at each depth for filtration and analysis of chlorophyll a (chl a). Mercury samples were transferred to a clean workspace outfitted with a HEPA filtered air-source and filtered onto 0.2 μm Nucleopore polycarbonate filters. Filters were dried in the hood, transferred to acid-cleaned petri dishes, and preserved at -20° C until analysis. Filtrate was poured off into 250/125 mL acid-cleaned glass bottles for analysis of total mercury, methylmercury, and dissolved organic carbon (DOC)/nutrients.  Mercury samples were acidified with 0.5%v/v hydrochloric acid (HCl) and refrigerated at 4° C, and DOC/nutrient samples were frozen. A separate aliquot of 500 – 1000 mL was filtered onto a 25 mm glass fiber filter for chl a analysis, and the filter was stored frozen in a petri dish, wrapped in aluminum foil. Samples for exploratory analysis of PFAS in seawater were also collected from the upper 250 m of the water column, transferred to HDPE bottles and stored at 4° C.

Sediment cores were collected using an MC-800 multi-corer, and a single core was sectioned and preserved at each site for Hg analysis. Prior to sectioning, S. Katz identified distinct horizons and recorded the position and Munsell color (Munsell Color Company, Inc., 2009). The top 20 cm of each Hg core was sectioned at intervals of 0.5 – 2 cm, with highest sampling density concentrated near the top of the core. This scheme is designed to maximize sampling resolution within the last 1000 years while leaving sufficient material for Hg stable isotopic analysis. Age models will be constructed for each core using radiocarbon dating of total organic carbon (to be performed by S. Katz), and this will be the basis for calculation of Hg accumulation rates (HgAR). Sectioning was performed using an aluminum sheet and plastic spoon, and samples have been stored in plastic containers and stored at -80° C until analysis.

Post-cruise Activities.  Seawater samples and particulate filters will be measured for total mercury, methylmercury, PFAS, DOC/nutrients, chlorophyll a, and total suspended solids (TSS) at the Biogeochemistry for Global Contaminants Lab in Cambridge, MA. These data will be interpreted alongside oceanographic parameters recorded by the CTD and detailed in Appendix D: CTD / Rosette Operations. Prior to analysis, sediment samples will be freeze-dried and homogenized. Concentration and thermal fractionation measurements will be performed by combustion using a direct mercury analyzer (Milestone DMA-80). Isotopic composition will be determined by MC-ICP-MS (Thermo Neptune) following isolation of Hg from sediment by combustion- or digestion-based matrix separation techniques.

Table II. Anticipated Measurements

| Medium | Measurement | n |
| --- | --- | --- |
| Seawater | Total mercury (Hg) | 60 |
|  | Methylmercury (MeHg) | 60 |
|  | PFAS | 30 |
|  | Dissolved organic carbon (DOC) | 60 |
|  | Chlorophyll a (Chl a) | 60 |
|  | Total suspended solids (TSS) | 60 |
| Sediment | Total mercury (Hg) | 180 |
|  | Mercury thermo-desorption (MTD) | 30-180 |
|  | Mercury stable isotopes | 30-180 |

See Appendix E for details of the CTD and Niskin water sample operations by the cruise participants from the Autonomous University of Barcelona.

### Surface Water Sampling (Katz, Lohmann)

Objectives.  The goal of surface water particle sampling was to collect recently deposited particulate black carbon across the equatorial Atlantic for source analysis.

Protocol.  During transits to and between sites, water from the shipboard centrifugal pump was filtered through quartz fiber filters (142 mm diameter). Quartz fiber filters had been baked at 450 ˚C and stored in combusted aluminum foil before being placed in the filter housing for a maximum of 12 hrs. Flow rate was measured at the start and end of each sample period and total flow was recorded with a flow meter. Samples were wrapped in combusted aluminum foil before being stored at -20 ˚C.

Table III: Surface water sampling information

| ID | Start Date/Time | End Date/Time | Sample Type | Initial Flow Rate (L/min) | End Flow Rate (L/min) | Total Flow (L) |
| --- | --- | --- | --- | --- | --- | --- |
| EN651-1W-QFF | 2/27/20 15:40 | 2/27/20 15:40 | FB | NA | NA | NA |
| EN651-2W-QFF | 2/27/20 15:44 | 2/27/20 21:00 | FS | 0.74 | 0.5 | 220.6 |
| EN651-3W-QFF | 2/27/20 21:09 | 2/28/20 2:07 | FS | 0.66 | 0.5 | 192.9 |
| EN651-4W-QFF | 2/28/20 2:23 | 2/28/20 10:18 | FS | 0.7 | 0.35 | 297.2 |
| EN651-5W-QFF | 2/28/20 10:24 | 2/28/20 15:16 | FS | 0.7 | 0.58 | 211 |
| EN651-6W-QFF | 2/28/20 15:34 | 2/28/20 20:54 | FS | 0.68 | 0.88 | 255.3 |
| EN651-7W-QFF | 2/28/20 21:07 | 2/29/20 1:42 | FS | 0.7 | 0.72 | 242.3 |
| EN651-8W-QFF | 2/29/20 2:03 | 2/29/20 10:07 | FS | 1.2 | 0.66 | 405.5 |
| EN651-9W-QFF | 2/29/20 10:16 | 2/29/20 13:50 | FS | 0.6 | 0.82 | 198.9 |
| EN651-10W-QFF | 2/29/20 14:03 | 2/29/20 19:39 | FS | 1.4 | 0.66 | 382.3 |
| EN651-11W-QFF | 2/29/20 19:50 | 3/1/20 0:47 | FS | 1.02 | 0.84 | 287.1 |
| EN651-12W-QFF | 3/1/20 0:57 | 3/1/20 9:42 | FS | 1.28 | 0.66 | 473.2 |
| EN651-13W-QFF | 3/1/20 9:48 | 3/1/20 12:42 | FS | 1.26 | 0.94 | 182.2 |
| EN651-14W-QFF | 3/2/20 0:41 | 3/2/20 10:17 | FS | 0.9 | 0.6 | 503.7 |
| EN651-15W-QFF | 3/2/20 10:23 | 3/2/20 12:29 | FS | 1.24 | 1.1 | 153.4 |
| EN651-16W-QFF | 3/2/20 12:37 | 3/2/20 12:37 | FB | NA | NA | NA |
| EN651-17W-QFF | 3/2/20 12:52 | 3/2/20 19:45 | FS | 1.38 | 1.04 | 503.2 |
| EN651-18W-QFF | 3/2/20 19:52 | 3/3/20 2:16 | FS | 1.4 | 0.94 | 475.5 |
| EN651-19W-QFF | 3/3/20 16:10 | 3/4/20 0:18 | FS | 1.38 | 0.44 | 508.7 |
| EN651-20W-QFF | 3/4/20 0:24 | 3/4/20 10:44 | FS | 0.7 | 0.6 | 457.2 |
| EN651-21W-QFF | 3/4/20 10:54 | 3/4/20 17:07 | FS | 1.36 | 1.02 | 449.3 |
| EN651-22W-QFF | 3/5/20 5:23 | 3/5/20 12:54 | FS | 1.22 | 1.08 | 430.4 |
| EN651-23W-QFF | 3/5/20 18:02 | 3/5/20 23:48 | FS | 1.38 | 1.04 | 430.4 |
| EN651-24W-QFF | 3/5/20 23:53 | 3/6/20 11:30 | FS | 0.52 | 0.98 | 426.3 |
| EN651-25W-QFF | 3/6/20 21:23 | 3/7/20 0:20 | FS | 1.2 | 0.94 | 188.9 |
| EN651-26W-QFF | 3/7/20 0:24 | 3/7/20 8:46 | FS | 1.36 | 0.68 | 503.6 |
| EN651-27W-QFF | 3/7/20 8:52 | 3/7/20 11:09 | FS | 1.2 | 0.9 | 323.3 |
| EN651-28W-QFF | 3/7/20 19:15 | 3/7/20 19:15 | FB | NA | NA | NA |
| EN651-29W-QFF | 3/7/20 21:49 | 3/8/20 9:20 | FS | 1.4 | 0.5 | 588.8 |
| EN651-30W-QFF | 3/8/20 9:36 | 3/8/20 19:15 | FS | 1.34 | 0.72 | 572.2 |
| EN651-31W-QFF | 3/9/20 2:36 | 3/9/20 10:33 | FS | 1.3 | 0.64 | 343.9 |
| EN651-32W-QFF | 3/9/20 10:38 | 3/9/20 19:53 | FS | 0.74 | 0.8 | 408.6 |
| EN651-33W-QFF | 3/10/20 1:41 | 3/10/20 9:30 | FS | 1.04 | 0.8 | 430.6 |
| EN651-34W-QFF | 3/10/20 9:36 | 3/10/20 12:43 | FS | 1.42 | 0.86 | 211 |
| EN651-35W-QFF | 3/10/20 12:50 | 3/10/20 12:50 | FB | NA | NA | NA |
| EN651-36W-QFF | 3/10/20 12:59 | 3/10/20 19:28 | FS | 0.82 | 0.5 | 297.8 |
| EN651-37W-QFF | 3/10/20 19:32 | 3/10/20 22:30 | FS | 1.4 | 1.4 | 189 |
| EN651-38W-QFF | 3/11/20 4:38 | 3/12/20 0:08 | FS | 1.4 | 0.3 | 747.4 |
| EN651-39W-QFF | 3/12/20 4:47 | 3/12/20 16:19 | FS | 1.08 | 0.6 | 634.7 |
| EN651-40W-QFF | 3/13/20 0:47 | 3/13/20 11:48 | FS | 0.7 | 0.8 | 502.4 |
| EN651-41W-QFF | 3/13/20 18:22 | 3/14/20 10:16 | FS | 1.6 | 0.5 | 809.5 |
| EN651-42W-QFF | 3/14/20 10:31 | 3/13/20 10:31 | FB | NA | NA | NA |
| EN651-43W-QFF | 3/14/20 19:48 | 3/15/20 9:57 | FS | 1.5 | 1.04 | 611.2 |
| EN651-44W-QFF | 3/16/20 10:16 | 3/16/20 20:05 | FS | 1.4 | 0.6 | 566.2 |
| EN651-45W-QFF | 3/16/20 20:18 | 3/17/20 8:30 | FS | 1 | 0.72 | 546.8 |

FS = Field Sample; FB = Field Blank

Post-cruise Activities.  Black carbon will be oxidized using the CTO-375 method for establishing concentrations and deriving the origin though 13C and ∆14C (Winiger et al. 2015; Zencak et al. 2007). Black carbon concentrations will also be tested using the thermo-optical Sunset method (Birch and Cary 1996), which is a routine measurement technique for aerosols, but has not been tried for aqueous particles. The origin of black carbon will also be assessed from the ratios of polycyclic aromatic hydrocarbons (PAHs) (Yunker et al. 2002) and levoglucosan concentrations, a tracer of softwood combustion (Simoneit et al. 1999; Simoneit and Elias 2000).

Sediment Sampling (Lohmann, Katz, Geyman, Rosell-Mele, Penalva Arias)

Objectives.  The objectives of collection and sectioning of sediment cores is to determine age and origin of black carbon in equatorial Atlantic sediments.

Protocol.  MC-800 tubes were labeled (EN651-“Site number”-MC”coring attempt number”“letter of core”,ex: EN651-01-MC01a) and photograph before sectioning. All cores and core sections were stored at -20 ˚C.

The water on top of the core was syphoned off and a thin piece of stainless-steel sheet was slid under the foot of the tube.  The foot was bent up and the stainless-steel sheet was used to transfer the core to the core extruder.  Cores were sectioned at 1 cm intervals down to 10 cm, then 2 cm intervals down to 20 cm, using the piece of stainless-steel and a cake spatula to cut them.  The remainder of the core was wrapped in combusted aluminum foil and placed in a zip-lock bag for storage.  Sections of cores were stored in amber glass jars placed in a freezer.  One core was transferred with the extruder to a PVC tube and capped for archival storage.  If 5 or more cores were recovered 0.5 cm sections would be taken down to 10 cm and the remainder of the core wrapped in foil and zip-lock bag before being frozen.  Due to limited supply of jars the 0.5 cm core sections were wrapped in combusted aluminum foil and placed in a ziplock bag before being stored with other samples.

Post-cruise Activities.  Black carbon will be oxidized using the CTO-375 method for establishing concentrations and deriving the origin though 13C and ∆14C (Winiger et al. 2015; Zencak et al. 2007). The sources of black carbon will also be derived from the ratio of polycyclic aromatic hydrocarbons (PAHs) (Yunker et al. 2002) and levoglucosan concentrations (Simoneit et al. 1999; Simoneit and Elias 2000).

### Air Sampling (Lohmann, Katz)

Objectives.  The goal of atmospheric particle sampling was to collect particulate black carbon across the equatorial Atlantic for source analysis.

Protocol.  During transits to and between sites hi-vol samplers were run on the roof of the pilothouse when the apparent wind was less than 45˚ off the bow.  Plywood was placed over the railing to increase airflow to the hi-vol sampler (Figure 4).  Quartz fiber filters baked at 450˚C and stored in combusted aluminum foil were installed in the hi-vol and run for a maximum of 24 hours when wind permitted.  Flow rate was measured using at the start and end of each sample period.  Sample were folded and wrapped in combusted aluminum foil before being stored at -20˚C.

Figure 4:  Photo of hi-vol sampler with plywood used to improve airflow to sampler

Table IV. Hi-vol sampling information

| ID | Start Date/Time | End Date/Time | Type | Average flow (m3/sec) | Total Flow Volume (m3) |
| --- | --- | --- | --- | --- | --- |
| EN651_01A_QFF | 2/27/20 14:15 | 2/27/20 14:15 | FB | NA | NA |
| EN651_02A_QFF | 2/27/20 13:35 | 2/28/20 14:20 | FS | 0.0172 | 1449.01 |
| EN651_03A_QFF | 2/28/20 14:35 | 2/29/20 13:16 | FS | 0.0172 | 1408.40 |
| EN651_04A_QFF | 2/29/20 13:22 | 3/01/20 11:51 | FS | 0.0172 | 1394.70 |
| EN651_05A_QFF | 3/02/20 00:23 | 3/03/20 00:53 | FS | 0.0183 | 1618.64 |
| EN651_06A_QFF | 3/03/20 16:01 | 3/04/20 16:37 | FS | 0.0213 | 1905.61 |
| EN651_07A_QFF | 3/04/20 18:40 | 3/04/20 23:54 | FS | 0.0204 | 393.97 |
| EN651_08A_QFF | 3/05/20 00:25 | 3/05/20 00:25 | FB | NA | NA |
| EN651_09A_QFF | 3/05/20 00:41 | 3/05/20 04:56 | FB | NA | NA |
| EN651_10A_QFF | 3/05/20 05:07 | 3/05/20 12:37 | FS | 0.0227 | 613.93 |
| EN651_11A_QFF | 3/05/20 17:51 | 3/06/20 11:14 | FS | 0.0209 | 1309.99 |
| EN651_12A_QFF | 3/06/20 20:38 | 3/07/20 10:26 | FS | 0.0209 | 1040.55 |
| EN651_13A_QFF | 3/07/20 10:33 | 3/07/20 10:33 | FB | NA | NA |
| EN651_14A_QFF | 3/07/20 10:58 | 3/07/20 10:58 | FB | NA | NA |
| EN651_15A_QFF | 3/07/20 21:34 | 3/08/20 19:01 | FS | 0.0227 | 1760.45 |
| EN651_16A_QFF | 3/09/20 02:20 | 3/09/20 19:42 | FS | 0.0232 | 1453.81 |
| EN651_17A_QFF | 3/10/20 01:22 | 3/10/20 20:20 | FS | 0.0232 | 1754.38 |
| EN651_18A_QFF | 3/11/20 04:35 | 3/11/20 23:56 | FS | 0.0234 | 1644.97 |
| EN651_19A_QFF | 3/12/20 04:35 | 3/12/20 18:09 | FS | 0.0236 | 1152.30 |
| EN651_20A_QFF | 3/13/20 00:31 | 3/13/20 11:37 | FS | 0.0233 | 933.78 |
| EN651_21A_QFF | 3/13/20 18:08 | 3/14/20 09:55 | FS | 0.0233 | 1324.49 |
| EN651_22A_QFF | 3/14/20 10:09 | 3/14/20 10:09 | FB | NA | NA |
| EN651_23A_QFF | 3/14/20 19:38 | 3/15/20 09:46 | FS | 0.0237 | 1207.68 |
| EN651_24A_QFF | 3/16/20 10:16 | 3/17/20 08:17 | FS | 0.0237 | 1876.31 |

FS = Field Sample; FB = Field Blank

Post-cruise Activities.  Black carbon will be oxidized using the CTO-375 method for establishing concentrations and deriving the origin though 13C and ∆14C (Winiger et al. 2015; Zencak et al. 2007). Black carbon concentrations will also be measured using the thermo-optical Sunset method (Birch and Cary 1996), which is a routine measurement technique for aerosols. The origin of black carbon will also be assessed from the ratios of polycyclic aromatic hydrocarbons (PAHs) (Yunker et al. 2002) and levoglucosan concentrations, a tracer of softwood combustion (Simoneit et al. 1999; Simoneit and Elias 2000).

References

Birch ME, Cary RA. 1996. Elemental Carbon-Based Method for Monitoring Occupational Exposures to Particulate Diesel Exhaust. Aerosol Sci Technol 25:221–241; doi:10.1080/02786829608965393.

Bond TC, Doherty SJ, Fahey DW, Forster PM, Berntsen T, Deangelo BJ, et al. 2013. Bounding the role of black carbon in the climate system: A scientific assessment. J Geophys Res Atmos 118:5380–5552; doi:10.1002/jgrd.50171.

Bond TC, Sun K. 2005. Can reducing black carbon emissions counteract global warming? Environ Sci Technol 39:5921–5926; doi:10.1021/es0480421.

Coppola AI, Wiedemeier DB, Galy V, Haghipour N, Hanke UM, Nascimento GS, et al. 2018. Global-scale evidence for the refractory nature of riverine black carbon. Nat Geosci https://doi.org/10.1038/s41561-018-0159-8; doi:10.1038/s41561-018-0159-8.

Divins, D.L., NGDC Total Sediment Thickness of the World's Oceans & Marginal Seas, June 20, 2009, http://www.ngdc.noaa.gov/mgg/sedthick/sedthick.html.

Dutkiewicz, A., Müller, R.D., O’Callaghan, S. and Jónasson, H., 2015. Census of seafloor sediments in the world’s ocean. Geology, 43(9), pp.795-798.

Elmquist M, Semiletov I, Guo L, Gustafsson Ö. 2008. Pan-Arctic patterns in black carbon sources and fluvial discharges deduced from radiocarbon and PAH source apportionment markers in estuarine surface sediments. Global Biogeochem Cycles 22:1–13; doi:10.1029/2007GB002994.

Gustafsson Ö̈, Ramanathan V. 2016. Convergence on climate warming by black carbon aerosols. Proc Natl Acad Sci 113:4243–4245; doi:10.1073/pnas.1603570113.

Hedges JI, Keil RG, Benner R. 1997. What happens to terrestrial organic matter in the ocean? Org Geochem 27:195–212; doi:10.1016/S0146-6380(97)00066-1.

Kuhlbusch TAJ. 1998. Black Carbon and the Carbon Cycle. Sci  280:1903–1904; doi:10.1126/science.280.5371.1903.

Masiello C a., Druffel ERM. 2001. Carbon isotope geochemistry of the Santa Clara River. Global Biogeochem Cycles 15:407; doi:10.1029/2000GB001290.

Mitra S, Bianchi TS, McKee BA, Sutula M. 2002. Black carbon from the Mississippi River: quantities, sources, and potential implications for the global carbon cycle. Environ Sci Technol 36:2296–2302; doi:10.1021/es015834b.

Mitra S, Zimmerman AR, Hunsinger GB, Woerner WR. 2014. Black carbon in coastal and large river systems,. In: Biogeochemical Dynamsics at Major River-Coastal Interfaces-Linkages with Climate Change. (T.S. Bianchi, M.A. Allison, and W.-J. Ca, eds). Cambridge University Press, New:New York, NY, USA. 200–234.

Müller, R.D., M. Sdrolias, C. Gaina, and W.R. Roest, 2008. Age, spreading rates and spreading         symmetry of the world's ocean crust, Geochem. Geophys. Geosyst., 9, Q04006.

Munsell Color Company, Inc., 2009. Munsell Soil Color Chart: (revised ed.): Newburgh, MD (Munsell Color).

Office of Water, US Environmental Protection Agency. Method 1669: Sampling Ambient Water for Trace Metals at EPA Water Quality Criteria Levels: Draft. Washington, DC: Office of Water, US Environmental Protection Agency; 1996.

Ramanathan V, Carmichael G. 2008. Global and Regional Climate Changes Due to Black Carbon. Nat Geosci 1: 221–227.

Simoneit BRT, Elias VO. 2000. Organic tracers from biomass burning in atmospheric particulate matter over the ocean. Mar Chem 69:301–312; doi:10.1016/S0304-4203(00)00008-6.

Simoneit BRT, Schauer JJ, Nolte CG, Oros DR, Elias VO, Fraser MP, et al. 1999. Levoglucosan, a tracer for cellulose in biomass burning and atmospheric particles. Atmos Environ 33:173–182; doi:10.1016/S1352-2310(98)00145-9.

Suman DO, Kuhlbusch TAJ, Lim B. 1997. Marine Sediments: A reservoir for black carbon and their use as spatial and temporal records of combustion. In: Sediment Records of Biomass Burning and Global Change (J.S. Clark, H. Cachier, J.G. Goldhammer, and B.J. Stocks., eds). Springer-Verlag. 271–293.

Winiger P, Andersson A, Yttri KE, Tunved P, Gustafsson Ö. 2015. Isotope-Based Source Apportionment of EC Aerosol Particles during Winter High-Pollution Events at the Zeppelin Observatory, Svalbard. Environ Sci Technol 49:11959–11966; doi:10.1021/acs.est.5b02644.

Yunker MB, Macdonald RW, Vingarzan R, Mitchell RH, Goyette D, Sylvestre S. 2002. PAHs in the Fraser River basin: a critical appraisal of PAH ratios as indicators of PAH source and composition. Org Geochem 33:489–515; doi:10.1016/S0146-6380(02)00002-5.

Zencak Z, Elmquist M, Gustafsson Ö. 2007. Quantification and radiocarbon source apportionment of black carbon in atmospheric aerosols using the CTO-375 method. Atmos Environ 41:7895–7906; doi:10.1016/j.atmosenv.2007.06.006.